USB TETHERING ON

LINUX SERVER

Complete Setup, Troubleshooting & Advanced Configuration Guide

Covers: Ubuntu · Debian · RHEL · Arch Linux

Android & iPhone Tethering  |  NetworkManager & systemd-networkd  |  Persistent Config

Version 1.0  |  April 2026

# 1. Introduction

USB tethering allows a smartphone (Android or iPhone) to share its mobile data connection with a Linux server over a USB cable. Unlike Wi-Fi or Bluetooth tethering, USB tethering is faster, more stable, does not drain the phone battery as quickly (the phone usually charges while connected), and does not require any wireless configuration on the server side.

This is particularly useful for:

- Headless servers with no Wi-Fi card

- Bootstrapping a server that has no network connection yet

- Fallback internet when your primary LAN/WAN is down

- Remote SSH access via Cloudflare Tunnel or similar, without a dedicated router

- Rapid deployment in a new location with just a phone and a USB cable

## 1.1 How It Works

When USB tethering is enabled on a phone, the phone presents itself to the Linux host as a USB network adapter (a RNDIS/CDC-ECM device on Android, or NCM on iPhone). Linux then sees a new network interface — typically named usb0, enp0s20u1, or similar — and can obtain an IP address via DHCP from the phone's internal DHCP server.

| Parameter | Typical Value | Notes |
| --- | --- | --- |
| Phone DHCP range | 192.168.42.x (Android) 172.20.10.x (iPhone) | Assigned by phone |
| Server IP | 192.168.42.2 (or similar) | Assigned automatically via DHCP |
| Gateway | 192.168.42.129 (Android) | Phone acts as NAT gateway |
| DNS | Google 8.8.8.8 or carrier DNS | Provided by phone |
| Interface name | usb0 / enp0s20u1 / eth1 | Depends on kernel & USB port |

# 2. Prerequisites

## 2.1 Hardware Requirements

- A Linux server (Ubuntu 20.04+, Debian 11+, RHEL/Rocky 8+, Arch, or similar)

- An Android smartphone (Android 4.0+) or iPhone (iOS 12+)

- A USB cable (preferably the original data cable — some cables are charge-only)

- USB port on the server (USB 2.0 or 3.0 — both work)

WARNING Always use a cable that supports data transfer. If tethering does not work, try a different cable first — this is a very common issue.

## 2.2 Software / Kernel Requirements

USB tethering is supported out-of-the-box on most modern Linux kernels (4.x+). The relevant kernel modules are:

| Module | Purpose | Phone Type |
| --- | --- | --- |
| rndis_host | Remote NDIS — most Android phones | Android |
| cdc_ether | CDC Ethernet — some Android phones | Android |
| cdc_ncm | Network Control Model — iPhones | iPhone (iOS) |
| ipheth | iPhone USB Ethernet — older iOS | iPhone (older) |
| usbnet | Core USB networking layer | All |

### Check if modules are available

Kernel Module Check

# Check if RNDIS module is available (Android) modinfo rndis_host # Check if CDC NCM module is available (iPhone) modinfo cdc_ncm # List all loaded modules related to USB networking lsmod ¦ grep -E 'rndis¦cdc¦ipheth¦usbnet'

# 3. Android USB Tethering Setup

## 3.1 Enable Tethering on Android Phone

- Connect your Android phone to the server via USB cable.

- On your phone, go to: Settings → Network & Internet → Hotspot & Tethering

- Toggle "USB tethering" to ON.

- If prompted, select "File Transfer" or "USB Tethering" mode (NOT "Charging Only").

INFO On some Samsung/MIUI devices, the path is: Settings → Connections → Mobile Hotspot and Tethering → USB Tethering

## 3.2 Verify the Interface Appears on Linux

Detecting the USB Tethering Interface

# Right after plugging in and enabling tethering on the phone: # Method 1: Check dmesg for the new USB device dmesg ¦ tail -30 # You should see lines like: # [12345.678] usb 1-1: new high-speed USB device number 3 using xhci_hcd # [12345.890] rndis_host 1-1:1.0 usb0: register 'rndis_host' # Method 2: List all network interfaces ip link show # Method 3: Look specifically for USB-related interfaces ip link show ¦ grep -E 'usb¦enp.*u' # Method 4: Use nmcli (NetworkManager) nmcli device status

## 3.3 Identify the Correct Interface Name

The interface name varies depending on your kernel version and systemd version:

- usb0  — older kernels / older naming scheme

- enp0s20u1  — typical on modern Ubuntu/Debian with predictable naming

- enp0s20u1u1  — if plugged into a USB hub

- eth1  — some minimal/embedded Linux setups

Identify New Interface

# Find the interface using its MAC address prefix or by watching changes # Run BEFORE plugging in: ip link show > /tmp/before.txt # Plug in phone and enable tethering, then run: ip link show > /tmp/after.txt # Compare the two outputs to see what's new: diff /tmp/before.txt /tmp/after.txt

## 3.4 Bring Up the Interface and Get IP via DHCP

### Using ip + dhclient (manual method)

Manual IP Assignment

# Replace 'usb0' with your actual interface name IFACE=usb0 # Bring the interface up sudo ip link set $IFACE up # Request an IP via DHCP from the phone sudo dhclient $IFACE # Verify you got an IP ip addr show $IFACE # Expected output: # 3: usb0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc fq_coda state UNKNOWN # link/ether xx:xx:xx:xx:xx:xx brd ff:ff:ff:ff:ff:ff # inet 192.168.42.167/24 brd 192.168.42.255 scope global dynamic usb0 # Test internet connectivity ping -c 4 8.8.8.8 curl -I https://google.com

### Using NetworkManager (recommended for desktops/servers with nmcli)

NetworkManager Method

# Check current device status nmcli device status # If the device shows 'unmanaged', set it to managed: nmcli device set usb0 managed yes # Connect using DHCP (NetworkManager auto-detects) nmcli device connect usb0 # Verify nmcli device show usb0 # Check routing ip route show

### Using systemd-networkd

systemd-networkd Method

# Create a .network file for the USB interface sudo nano /etc/systemd/network/25-usb-tethering.network # Paste the following content: # ----------------------------------------------- # [Match] # Name=usb* # Driver=rndis_host # # [Network] # DHCP=yes # ----------------------------------------------- # Restart networkd sudo systemctl restart systemd-networkd # Check status networkctl status networkctl list

# 4. iPhone USB Tethering Setup

## 4.1 Enable Tethering on iPhone

- Go to Settings → Personal Hotspot on your iPhone.

- Enable "Allow Others to Join".

- Connect iPhone to Linux server via USB cable.

- On iPhone, when prompted, tap "Trust" for the computer.

WARNING iPhone USB tethering requires the ipheth or cdc_ncm kernel module. Most modern Linux kernels include this, but some minimal/server kernels may not.

## 4.2 Install Required Packages

iPhone Dependencies

# Ubuntu / Debian sudo apt update sudo apt install -y libimobiledevice-utils usbmuxd ifuse # RHEL / Rocky / AlmaLinux / Fedora sudo dnf install -y libimobiledevice usbmuxd # Arch Linux sudo pacman -S libimobiledevice usbmuxd # Enable usbmuxd service sudo systemctl enable --now usbmuxd

## 4.3 Pair the iPhone

iPhone Pairing

# List connected iOS devices idevice_id -l # Pair with the device (tap Trust on iPhone when prompted) idevicepair pair # Verify pairing idevicepair validate # Expected output: SUCCESS: Paired with device ...

## 4.4 Bring Up the Interface

Bring Up iPhone Interface

# After pairing, a network interface should appear ip link show ¦ grep -E 'eth¦enp' # The iPhone interface is usually eth1 or enp0s20u1 # Enable it and get an IP via DHCP sudo dhclient eth1 # replace eth1 with your interface # Verify ip addr show eth1 ping -c 3 8.8.8.8

# 5. Making USB Tethering Persistent (Auto-Connect)

By default, you need to run dhclient manually every time you plug in the phone. Here is how to make it automatic and persistent across reboots.

## 5.1 Using udev Rules (Universal Method)

udev rules trigger automatically when a USB device is plugged in. This is the most reliable way to auto-configure tethering.

Find Device IDs

# First, find the vendor and product IDs of your phone # Plug in the phone with tethering enabled, then run: lsusb # Example output: # Bus 002 Device 003: ID 04e8:6864 Samsung Electronics Co., Ltd # ^^^^ ^^^^ # vendor_id product_id # Another way — watch for USB events: udevadm monitor --udev --subsystem-match=usb/usb_device # (plug phone in now and check output) # Get detailed attributes of the device: udevadm info -a -p $(udevadm info -q path -n /dev/bus/usb/002/003)

/etc/udev/rules.d/90-usb-tethering.rules

# Create the udev rule file sudo nano /etc/udev/rules.d/90-usb-tethering.rules # ----------------------------------------------- # Paste this content (replace XXXX with your vendor ID): SUBSYSTEM=="net", ACTION=="add", DRIVERS=="rndis_host", \ ATTR{address}!="", \ RUN+="/usr/local/bin/usb-tether-up.sh $name" # For iPhone, use cdc_ncm or ipheth instead of rndis_host: SUBSYSTEM=="net", ACTION=="add", DRIVERS=="cdc_ncm", \ ATTR{address}!="", \ RUN+="/usr/local/bin/usb-tether-up.sh $name" # -----------------------------------------------

Helper Script + Reload udev

# Create the helper script sudo nano /usr/local/bin/usb-tether-up.sh # ----------------------------------------------- #!/bin/bash IFACE=$1 LOG="/var/log/usb-tether.log" echo "$(date): USB tether interface $IFACE appeared" >> $LOG # Bring up interface ip link set $IFACE up # Wait for interface to stabilize sleep 2 # Request IP via DHCP dhclient -v $IFACE >> $LOG 2>&1 echo "$(date): dhclient done for $IFACE" >> $LOG # Optional: add a default route via this interface # ip route add default dev $IFACE metric 200 # ----------------------------------------------- # Make executable sudo chmod +x /usr/local/bin/usb-tether-up.sh # Reload udev rules sudo udevadm control --reload-rules sudo udevadm trigger

## 5.2 Using NetworkManager (Easiest on Ubuntu)

/etc/NetworkManager/system-connections/USB-Tethering.nmconnection

# NetworkManager can be configured to auto-manage USB tether interfaces # Create a connection profile for USB tethering sudo nmcli connection add \ type ethernet \ con-name 'USB-Tethering' \ ifname '*' \ connection.autoconnect yes \ ipv4.method auto \ ipv6.method ignore # Or create the config file directly: sudo nano /etc/NetworkManager/system-connections/USB-Tethering.nmconnection # ----------------------------------------------- [connection] id=USB-Tethering type=ethernet autoconnect=true autoconnect-priority=10 [ethernet] wake-on-lan=default [ipv4] method=auto [ipv6] addr-gen-mode=stable-privacy method=ignore # ----------------------------------------------- # Set correct permissions (required!) sudo chmod 600 /etc/NetworkManager/system-connections/USB-Tethering.nmconnection # Reload sudo nmcli connection reload

## 5.3 Using systemd Service for Auto-Reconnect

/etc/systemd/system/usb-tether.service

# Create a systemd service that monitors for USB tether sudo nano /etc/systemd/system/usb-tether.service # ----------------------------------------------- [Unit] Description=USB Tethering Auto-Connect After=network.target Wants=network.target [Service] Type=oneshot RemainAfterExit=yes ExecStart=/usr/local/bin/usb-tether-up.sh usb0 ExecStop=/sbin/ip link set usb0 down Restart=on-failure [Install] WantedBy=multi-user.target # ----------------------------------------------- sudo systemctl daemon-reload sudo systemctl enable usb-tether.service sudo systemctl start usb-tether.service # Check status sudo systemctl status usb-tether.service

# 6. Routing & DNS Configuration

## 6.1 Check Default Route

Routing Table

# View current routing table ip route show # Example output with USB tether as default: # default via 192.168.42.129 dev usb0 proto dhcp src 192.168.42.167 # 192.168.42.0/24 dev usb0 proto kernel scope link src 192.168.42.167 # If no default route exists via USB, add one manually: sudo ip route add default via 192.168.42.129 dev usb0 # To give USB tether a higher priority (lower metric = higher priority): sudo ip route add default via 192.168.42.129 dev usb0 metric 100

## 6.2 DNS Configuration

DNS Setup

# Check current DNS servers cat /etc/resolv.conf # If DNS is missing, add manually: sudo nano /etc/resolv.conf # ----------------------------------------------- nameserver 8.8.8.8 nameserver 8.8.4.4 nameserver 1.1.1.1 # ----------------------------------------------- # On Ubuntu with systemd-resolved: sudo systemd-resolve --interface=usb0 --set-dns=8.8.8.8 # Or configure resolvconf: echo 'nameserver 8.8.8.8' ¦ sudo resolvconf -a usb0.dhcp # Test DNS resolution dig google.com @8.8.8.8 nslookup google.com

## 6.3 Multiple Interfaces — Policy-Based Routing

If your server has both a LAN connection and USB tethering, you may want to route specific traffic through the phone (e.g., SSH fallback). Here is how to set up policy-based routing:

Policy-Based Routing

# Add a separate routing table for USB tether echo '200 usb_tether' ¦ sudo tee -a /etc/iproute2/rt_tables # Add routes to the custom table sudo ip route add default via 192.168.42.129 dev usb0 table usb_tether sudo ip route add 192.168.42.0/24 dev usb0 scope link table usb_tether # Add a rule: packets FROM the USB interface use the usb_tether table sudo ip rule add from 192.168.42.167 table usb_tether priority 100 # Verify ip rule show ip route show table usb_tether

# 7. Troubleshooting

## 7.1 Interface Does Not Appear

Interface Not Found

# Step 1: Check if phone is detected at USB level lsusb # Step 2: Check kernel messages dmesg ¦ tail -50 ¦ grep -iE 'usb¦rndis¦tether¦cdc' # Step 3: Load the RNDIS module manually sudo modprobe rndis_host sudo modprobe cdc_ether # For iPhone: sudo modprobe cdc_ncm sudo modprobe ipheth # Step 4: Check if module loaded lsmod ¦ grep rndis # Step 5: Check USB device details usb-devices ¦ grep -A 20 'Vendor'

WARNING The most common reason is the phone is in 'Charging only' mode. Unlock your phone, swipe down the notification bar, tap the USB notification, and select 'File Transfer' or 'USB Tethering'.

## 7.2 Interface Appears but No IP Address

No IP Address

# Check interface state ip link show usb0 # If DOWN, bring it up first sudo ip link set usb0 up # Try dhclient with verbose output sudo dhclient -v usb0 # Or use dhcpcd instead sudo apt install dhcpcd5 sudo dhcpcd usb0 # Check for DHCP errors sudo journalctl -u dhclient -n 50 sudo journalctl -u NetworkManager -n 50 ¦ grep -i usb # Check that the phone's hotspot is actually active # Verify by checking phone screen — hotspot icon should be visible

## 7.3 IP Obtained but No Internet

Internet Not Working

# Check routing table ip route show # Ping the phone's gateway directly ping -c 3 192.168.42.129 # If gateway is reachable but internet is not: # Check if the default route points to usb0 ip route get 8.8.8.8 # If default route is wrong, fix it: sudo ip route del default sudo ip route add default via 192.168.42.129 dev usb0 # Test DNS ping -c 3 8.8.8.8 # Direct IP — tests routing ping -c 3 google.com # Domain name — tests DNS too # Check if phone has mobile data (obvious but worth checking!) # Check phone's mobile data toggle

## 7.4 Connection Drops After a While

Connection Drops Fix

# Keep-alive: prevent DHCP lease expiry issues # Add a cron job to renew DHCP periodically sudo crontab -e # Add this line (renew every 10 minutes): */10 * * * * /sbin/dhclient usb0 >> /var/log/usb-tether.log 2>&1 # Check for USB power management issues cat /sys/bus/usb/devices/*/power/control # Disable USB autosuspend for the phone's USB port # Find the correct USB device path from lsusb first echo 'on' ¦ sudo tee /sys/bus/usb/devices/1-1/power/control # Permanent fix via udev: sudo nano /etc/udev/rules.d/91-usb-power.rules # Add: ACTION=="add", SUBSYSTEM=="usb", ATTR{idVendor}=="04e8", \ ATTR{power/control}="on" # (replace 04e8 with your phone's vendor ID)

## 7.5 iPhone: 'Trust This Computer' Not Appearing

iPhone Trust Issue

# Ensure usbmuxd is running sudo systemctl status usbmuxd sudo systemctl start usbmuxd # Unpair and re-pair idevicepair unpair # Unplug iPhone, wait 5 seconds, replug # Unlock iPhone before plugging # Then pair again: idevicepair pair # If still not working, reset iPhone trust settings: # iPhone: Settings → General → Transfer or Reset iPhone → Reset → Reset Location & Privacy # Check libimobiledevice version ideviceinfo --version # Update if needed sudo apt upgrade libimobiledevice-utils

## 7.6 Diagnostic Commands Reference

| Command | What It Tells You |
| --- | --- |
| lsusb | Lists all USB devices — confirms phone is detected |
| dmesg ¦ tail -30 | Kernel messages — shows USB events and errors |
| ip link show | All network interfaces and their state |
| ip addr show | IP addresses assigned to each interface |
| ip route show | Routing table — shows how traffic is routed |
| nmcli device status | NetworkManager device states |
| dhclient -v usb0 | Verbose DHCP — shows lease negotiation details |
| journalctl -u NetworkManager -f | Live NetworkManager logs |
| ping -c 3 192.168.42.129 | Test connectivity to phone (gateway) |
| cat /etc/resolv.conf | Current DNS server configuration |

# 8. Advanced Topics

## 8.1 SSH Access from Outside via Cloudflare Tunnel over USB Tether

Once your server has internet via USB tethering, you can expose SSH anywhere using Cloudflare Tunnel — no port forwarding or static IP required.

Cloudflare Tunnel over USB Tether

# Install cloudflared curl -L https://pkg.cloudflare.com/cloudflare-main.gpg \ ¦ sudo tee /usr/share/keyrings/cloudflare-main.gpg > /dev/null echo "deb [signed-by=/usr/share/keyrings/cloudflare-main.gpg] \ https://pkg.cloudflare.com/cloudflared $(lsb_release -cs) main" \ ¦ sudo tee /etc/apt/sources.list.d/cloudflared.list sudo apt update && sudo apt install cloudflared # Authenticate cloudflared tunnel login # Create tunnel cloudflared tunnel create my-server # Expose SSH cloudflared tunnel --url ssh://localhost:22 # Access from your laptop: # ssh -o ProxyCommand='cloudflared access ssh --hostname YOUR_TUNNEL_URL' user@localhost

## 8.2 Share USB Tether Internet to LAN (IP Masquerading)

If your server is connected to a LAN and you want to share the phone's internet with other LAN devices:

Share Internet to LAN

# Enable IP forwarding echo 'net.ipv4.ip_forward=1' ¦ sudo tee -a /etc/sysctl.conf sudo sysctl -p # Set up NAT (masquerade) # usb0 = phone-connected interface # eth0 = LAN interface sudo iptables -t nat -A POSTROUTING -o usb0 -j MASQUERADE sudo iptables -A FORWARD -i eth0 -o usb0 -j ACCEPT sudo iptables -A FORWARD -i usb0 -o eth0 -m state \ --state RELATED,ESTABLISHED -j ACCEPT # Make iptables rules persistent sudo apt install iptables-persistent sudo netfilter-persistent save # LAN clients need to use this server as their gateway: # Gateway: <this server's LAN IP> (e.g., 192.168.1.100)

## 8.3 Monitoring & Bandwidth

Bandwidth Monitoring

# Monitor real-time bandwidth on USB tether interface sudo apt install nload nload usb0 # Or use iftop for per-connection breakdown sudo apt install iftop sudo iftop -i usb0 # Check total data used (useful for limited mobile data): sudo apt install vnstat sudo vnstat -i usb0 sudo vnstat -i usb0 --live # One-liner to see current transfer rates: watch -n 1 'cat /proc/net/dev ¦ grep usb'

## 8.4 Making Modules Load at Boot

Module Auto-Load at Boot

# If rndis_host module doesn't load automatically at boot: # Add to /etc/modules echo 'rndis_host' ¦ sudo tee -a /etc/modules echo 'cdc_ether' ¦ sudo tee -a /etc/modules # For iPhone: echo 'cdc_ncm' ¦ sudo tee -a /etc/modules echo 'ipheth' ¦ sudo tee -a /etc/modules # Alternative: modprobe config sudo nano /etc/modprobe.d/usb-tethering.conf # Add: # options rndis_host # Update initramfs to include modules sudo update-initramfs -u # Verify modules are loaded after reboot lsmod ¦ grep -E 'rndis¦cdc¦ipheth'

# 9. Quick Reference Cheat Sheet

| Scenario | Command |
| --- | --- |
| Detect new interface after plug-in | dmesg ¦ tail -20 |
| List all interfaces | ip link show |
| Bring interface up | sudo ip link set usb0 up |
| Get IP via DHCP | sudo dhclient usb0 |
| Check IP address | ip addr show usb0 |
| Check routing | ip route show |
| Add default route | sudo ip route add default via 192.168.42.129 dev usb0 |
| Test internet (IP) | ping -c 3 8.8.8.8 |
| Test internet (DNS) | ping -c 3 google.com |
| Release DHCP lease | sudo dhclient -r usb0 |
| Renew DHCP lease | sudo dhclient usb0 |
| Remove interface config | sudo ip addr flush dev usb0 |
| Monitor bandwidth | nload usb0 |
| Check NetworkManager | nmcli device status |
| Restart NetworkManager | sudo systemctl restart NetworkManager |
| Live kernel USB events | udevadm monitor --subsystem-match=net |
| USB power management off | echo on ¦ sudo tee /sys/bus/usb/devices/1-1/power/control |
| Load RNDIS module | sudo modprobe rndis_host |
| Load iPhone module | sudo modprobe cdc_ncm |
| Check module loaded | lsmod ¦ grep rndis |

# 10. Summary

USB tethering is one of the simplest and most reliable ways to get internet on a Linux server without Wi-Fi. Here is a condensed flow of what to do:

- Connect phone → Enable USB tethering on phone

- Run: dmesg | tail -20  to see the new interface name

- Run: sudo ip link set usb0 up

- Run: sudo dhclient usb0

- Run: ping -c 3 8.8.8.8  to verify

- Optionally set up udev rules or NetworkManager for auto-connect

TIP For a permanent headless server setup, the udev + helper script method (Section 5.1) combined with systemd-networkd (Section 3.4) is the most robust approach. It will auto-connect whenever the phone is plugged in and tethering is enabled, even after reboots.

With USB tethering configured, you can combine it with Cloudflare Tunnel (Section 8.1) to maintain full remote SSH access from anywhere in the world using just a smartphone data plan — no ISP, no router, no static IP required.

— End of Document —